# Frame Aligner Verification Plan

Author: Adam Salameh
Project: Frame Aligner Verification

## 1. Objective

The purpose of this verification plan is to define the strategy, coverage, and corner-case testing required to validate the Frame Aligner design. The primary goal is to ensure accurate header detection, stable synchronization, and correct recovery from illegal frames.

## 2. Design Features to be Verified

| Feature | Description |
| --- | --- |
| Frame Header Detection | Detects valid headers (AFAA, BA55) and rejects illegal ones. |
| Frame Synchronization | Ensures frame_detect and fr_byte_position react properly to frame validity. |
| Payload Processing | Verifies payload bytes do not trigger false header detection. |
| Reset Behavior | Checks that all counters and internal logic reset properly. |
| Error Recovery | Ensures the DUT recovers from invalid frames cleanly. |

## 3. Verification Environment Overview

| Component | Purpose |
| --- | --- |
| Interface | Connects DUT and testbench signals (rx_data, clk, reset, frame_detect, fr_byte_position). |
| Generator | Creates random, legal, illegal and directed test frames. |
| Driver | Drives DUT input and gathers coverage data. |
| Monitor | Observes DUT activity for checking. |
| Scoreboard | Compares expected vs. actual DUT behavior. |
| Testbench | Integrates all components with DUT. |

## 4. Verification Methodology

The verification approach includes constrained-random testing, directed corner-case tests, and functional coverage measurement. A self-checking testbench monitors DUT signals and validates expected frame alignment behavior using assertions and a scoreboard.

## 5. Functional Coverage Plan

| Coverage Type | Description | Goal |
| --- | --- | --- |
| Header Coverage | Covers all legal and illegal headers, including cross coverage of MSB/LSB. | 100% |
| Payload Coverage | Covers header-like and reversed patterns appearing inside payloads. | 100% |
| Overall Functional Coverage | Aggregate of header and payload coverage. | ≥98% |

## 6. Corner Cases and Directed Tests

| ID | Description | Expected Behavior | Status |
| --- | --- | --- | --- |
| CC1 | Half-valid header (LSB valid, MSB invalid) | fr_byte_position toggles for one cycle | ✅ |
| CC2 | Half-valid header (LSB invalid, MSB valid) | fr_byte_position toggles for one cycle | ✅ |
| CC3 | HEAD1 LSB + HEAD2 MSB | fr_byte_position toggles for one cycle | ✅ |
| CC4 | HEAD2 LSB + HEAD1 MSB | fr_byte_position toggles for one cycle | ✅ |
| CC5 | HEAD2 payload includes AFAA | frame detection ignored the payload | ✅ |
| CC6 | ILLEGAL header, payload includes AAAF (reversed) | false detection | ✅ |
| CC7 | HEAD1 payload includes BA55 | frame detection ignored the payload | ✅ |
| CC8 | ILLEGAL header, payload includes 55BA (reversed) | false detection | ✅ |
| CC9 | 4 Continuous illegal headers | Frame _detect changed to 0 | ✅ |
| CC10 | 3 Continuous legal headers | Frame _detect changed to 1 | ✅ |
| CC11 | Mixed valid/invalid bursts | Stable operation | ✅ |

## 7. Observed Design Issues

- Issue 1: When the header is half-valid (LSB valid, MSB invalid), fr_byte_position briefly toggles to 1 then goes back to normal operation.

- Issue 2: When an illegal frame header is received, but the payload contains a reversed legal header pattern, the DUT incorrectly interprets it as the start of a new frame instead of ignoring it as part of the payload.

## 8. Verification Metrics

| Metric | Target | Achieved |
| --- | --- | --- |
| Functional Coverage | 100% | 72.771% |
| Header Coverage | 100% | 53.181% |
| Payload Coverage | 100% | 92.361% |